# ROS PACKAGE COMMANDS

# 1. Boot Ubuntu and verify Humble

Open Ubuntu Terminal:

Run:

lsb_release -sc
source /opt/ros/humble/setup.bash
echo $ROS_DISTRO

Expected:

jammy
humble

If /opt/ros/humble/setup.bash does not exist, stop there: ROS2 Humble is not installed and needs installation before your project can work.

Add Humble sourcing permanently:

grep -qxF 'source /opt/ros/humble/setup.bash' ~/.bashrc || \
echo 'source /opt/ros/humble/setup.bash' >> ~/.bashrc

# 2. Install system dependencies

Run:

sudo apt update
sudo apt install -y \
  python3-pip \
  python3-colcon-common-extensions \
  python3-rosdep \
  git \
  python3-numpy \
  python3-opencv \
  ros-humble-cv-bridge \
  ros-humble-image-transport \
  ros-humble-diagnostic-msgs \
  ros-humble-visualization-msgs \
  ros-humble-rviz2 \
  ros-humble-rqt-image-view \
  ros-humble-realsense2-camera

Initialize rosdep safely:

if [ ! -f /etc/ros/rosdep/sources.list.d/20-default.list ]; then
  sudo rosdep init
fi

rosdep update

Install Ultralytics:

python3 -m pip install --user ultralytics

Verify:

python3 -c "from ultralytics import YOLO; print('Ultralytics OK')"

Expected:

Ultralytics OK

# 3. Transfer your existing folder

If using a USB drive, plug it in and run:

ls /media/$USER

You will see a folder name, for example:

MY_USB

Copy your project:

cp -r /media/$USER/MY_USB/hrc_proximity_final ~/

Replace MY_USB with the real USB folder name.

Verify:

ls ~/hrc_proximity_final
ls ~/hrc_proximity_final/models

You must see:

ros2_node.py
config.py
detector.py
depth_source.py
proximity.py
dwell_tracker.py
overlay.py
logger.py
screenshot.py
audio_alert.py
requirements.txt
best_yolo11s.pt

If you cloned from GitHub, model weights are normally excluded. In that case, manually copy:

best_yolo11s.pt

into:

~/hrc_proximity_final/models/

GitHub blocks regular Git files larger than 100 MiB, so excluding the model unless you use Git LFS is appropriate.

# 4. Create the proper ROS2 package

Copy-paste this entire block:

source /opt/ros/humble/setup.bash

mkdir -p ~/hrc_ws/src
cd ~/hrc_ws/src

ros2 pkg create hrc_proximity_monitor \
  --build-type ament_python \
  --dependencies rclpy sensor_msgs std_msgs diagnostic_msgs visualization_msgs cv_bridge

cd ~/hrc_ws/src/hrc_proximity_monitor

mkdir -p models launch

Now ROS has created the formal package structure.

# 5. Copy the final files in

Copy-paste:

OLD_PROJECT="$HOME/hrc_proximity_final"
PACKAGE="$HOME/hrc_ws/src/hrc_proximity_monitor"
PYTHON_PACKAGE="$PACKAGE/hrc_proximity_monitor"

cp "$OLD_PROJECT/ros2_node.py" "$PYTHON_PACKAGE/"
cp "$OLD_PROJECT/config.py" "$PYTHON_PACKAGE/"
cp "$OLD_PROJECT/detector.py" "$PYTHON_PACKAGE/"
cp "$OLD_PROJECT/depth_source.py" "$PYTHON_PACKAGE/"
cp "$OLD_PROJECT/proximity.py" "$PYTHON_PACKAGE/"
cp "$OLD_PROJECT/dwell_tracker.py" "$PYTHON_PACKAGE/"
cp "$OLD_PROJECT/overlay.py" "$PYTHON_PACKAGE/"
cp "$OLD_PROJECT/logger.py" "$PYTHON_PACKAGE/"
cp "$OLD_PROJECT/screenshot.py" "$PYTHON_PACKAGE/"
cp "$OLD_PROJECT/audio_alert.py" "$PYTHON_PACKAGE/"
cp "$OLD_PROJECT/requirements.txt" "$PACKAGE/"
cp "$OLD_PROJECT/models/best_yolo11s.pt" "$PACKAGE/models/"

Verify:

find "$PACKAGE" -maxdepth 2 -type f | sort

You should see source code inside:

.../hrc_proximity_monitor/hrc_proximity_monitor/

and the model here:

.../hrc_proximity_monitor/models/best_yolo11s.pt

# 6. Confirm source imports

Your final files must already use relative imports. Check:

grep -RInE \
'^(import config|from (detector|depth_source|proximity|dwell_tracker|logger|overlay|screenshot|audio_alert) import)' \
"$PYTHON_PACKAGE"

Expected result:

No output

If it shows any lines, stop and send the output rather than guessing.

You do not need to run the older sed command because the final code I gave you already uses from . import config and other relative imports.

# 7. Replace package setup files

## Create setup.py

Copy-paste this command exactly, then replace YOUR_NAME and YOUR_EMAIL@example.com afterward if needed:

cat > ~/hrc_ws/src/hrc_proximity_monitor/setup.py <<'PY'
from glob import glob
import os

from setuptools import find_packages, setup


package_name = "hrc_proximity_monitor"


setup(
    name=package_name,
    version="0.0.1",
    packages=find_packages(exclude=["test"]),
    data_files=[
        (
            "share/ament_index/resource_index/packages",
            ["resource/" + package_name],
        ),
        (
            "share/" + package_name,
            ["package.xml"],
        ),
        (
            os.path.join("share", package_name, "launch"),
            glob("launch/*.launch.py"),
        ),
        (
            os.path.join("share", package_name, "models"),
            glob("models/*.pt"),
        ),
    ],
    install_requires=["setuptools"],
    zip_safe=True,
    maintainer="YOUR_NAME",
    maintainer_email="YOUR_EMAIL@example.com",
    description=(
        "Real-time depth-aware human detection and proximity warning "
        "system using YOLO and an RGB-D camera."
    ),
    license="Apache-2.0",
    entry_points={
        "console_scripts": [
            "proximity_monitor = hrc_proximity_monitor.ros2_node:main",
        ],
    },
)
PY

## Create setup.cfg

cat > ~/hrc_ws/src/hrc_proximity_monitor/setup.cfg <<'EOF'
[develop]
script_dir=$base/lib/hrc_proximity_monitor

[install]
install_scripts=$base/lib/hrc_proximity_monitor
EOF

## Create package.xml

cat > ~/hrc_ws/src/hrc_proximity_monitor/package.xml <<'EOF'
<?xml version="1.0"?>
<?xml-model href="http://download.ros.org/schema/package_format3.xsd"
  schematypens="http://www.w3.org/2001/XMLSchema"?>

<package format="3">
  <name>hrc_proximity_monitor</name>
  <version>0.0.1</version>

  <description>
    Real-time depth-aware human detection and proximity warning system
    using YOLO and an RGB-D camera.
  </description>

  <maintainer email="YOUR_EMAIL@example.com">YOUR_NAME</maintainer>
  <license>Apache-2.0</license>

  <buildtool_depend>ament_python</buildtool_depend>

  <depend>rclpy</depend>
  <depend>sensor_msgs</depend>
  <depend>std_msgs</depend>
  <depend>diagnostic_msgs</depend>
  <depend>visualization_msgs</depend>
  <depend>cv_bridge</depend>

  <exec_depend>python3-numpy</exec_depend>
  <exec_depend>python3-opencv</exec_depend>

  <export>
    <build_type>ament_python</build_type>
  </export>
</package>
EOF

## Create launch file

cat > ~/hrc_ws/src/hrc_proximity_monitor/launch/proximity_monitor.launch.py <<'PY'
from launch import LaunchDescription
from launch_ros.actions import Node


def generate_launch_description():
    return LaunchDescription(
        [
            Node(
                package="hrc_proximity_monitor",
                executable="proximity_monitor",
                name="ros2_proximity_node",
                output="screen",
            )
        ]
    )
PY

These setup files make your node executable through the ROS2 build system, rather than as a loose Python script. ROS2's standard workflow is colcon build followed by running the installed package executable.

# 8. Build and validate before camera

Run:

cd ~/hrc_ws
source /opt/ros/humble/setup.bash

rosdep install --from-paths src --ignore-src -r -y
colcon build --symlink-install
source install/setup.bash

Then run all three checks:

ros2 pkg list | grep '^hrc_proximity_monitor$'
ros2 pkg executables hrc_proximity_monitor

test -f \
~/hrc_ws/install/hrc_proximity_monitor/share/hrc_proximity_monitor/models/best_yolo11s.pt \
&& echo "Model installed correctly" \
|| echo "MODEL MISSING"

Correct output includes:

hrc_proximity_monitor
hrc_proximity_monitor proximity_monitor
Model installed correctly

If all three appear, then you have:

✓ Proper ROS2 Humble package
✓ Working package structure
✓ ros2_node.py installed as your ROS2 wrapper
✓ YOLO model installed in ROS package share directory

# 9. Camera testing

Only do this when the D435i is connected with a USB 3 port.

## Terminal 1 — RealSense camera driver

source /opt/ros/humble/setup.bash

ros2 launch realsense2_camera rs_launch.py \
  enable_color:=true \
  enable_depth:=true \
  enable_sync:=true \
  align_depth.enable:=true \
  depth_module.depth_profile:=640x480x30 \
  rgb_camera.color_profile:=640x480x30

Leave it running.

## Terminal 2 — confirm topics

source /opt/ros/humble/setup.bash

ros2 topic list | grep camera

Look specifically for:

/camera/camera/color/image_raw
/camera/camera/aligned_depth_to_color/image_raw
/camera/camera/color/camera_info

Also check frequency:

ros2 topic hz /camera/camera/color/image_raw
ros2 topic hz /camera/camera/aligned_depth_to_color/image_raw

You want roughly 30 Hz. The RealSense ROS2 wrapper uses align_depth.enable:=true to publish depth aligned to the RGB stream.

## Terminal 3 — run your wrapper package

source /opt/ros/humble/setup.bash
source ~/hrc_ws/install/setup.bash

ros2 launch hrc_proximity_monitor proximity_monitor.launch.py

Expected:

An OpenCV window opens.

## Terminal 4 — verify ROS outputs

source /opt/ros/humble/setup.bash
source ~/hrc_ws/install/setup.bash

ros2 topic echo /safety_status

Optional checks:

ros2 topic echo /human_distance
ros2 topic echo /diagnostics
ros2 topic list | grep -E 'camera_overlay|human_markers'

# 10. RViz

After your OpenCV window works:

source /opt/ros/humble/setup.bash
source ~/hrc_ws/install/setup.bash
rviz2

In RViz:

Set Global Options → Fixed Frame to the camera frame visible in the camera topics—usually:

camera_color_optical_frame

Click Add → By topic → select:

/camera_overlay

Click Add → By topic → select:

/human_markers

What you should see:

/camera_overlay
  Live RGB image with boxes, distance, XYZ, FPS,
  and PROXIMITY BREACH text.

/human_markers
  Green sphere = safe person.
  Red sphere = confirmed proximity breach.
  Text shows track ID, XYZ coordinate, and zone.